NC-5336									May 2026

JCB SINGLE DRUM SOIL COMPACTORS DELIVER STAGE V POWER

JCB is building on the launch of Europe’s first 12-tonne, Stage V, single-drum soil compactor last year, by unveiling a further two models competing in the 12-15 tonne sector. The highly efficient VM118D is joined by the larger VM128D and the VM138D, extending JCB’s leadership in this competitive area of the compaction market.

Key features include:

- Modular design with high degree of component commonality

- Smooth drum initially, with padfoot drums to follow

- 256kN-292kN of centrifugal force delivering superior compaction with fewer passes

- High torque engines with four traction modes and balanced hydraulics

- Air-conditioned ROPS/FOPS cabin package available

Clean compaction

JCB has unveiled the 12.1-tonne VM128D and the 13.2-tonne VM138D, to sit alongside the 11.9-tonne VM118D launched in 2025. These three single drum soil compactors use a modular construction to create a truly global compaction platform. All three machines are powered by Stage V compliant JCB diesel engines.

The VM118D uses JCB’s 430 diesel engine, delivering 55kW, achieving Stage V without any requirement for Diesel Exhaust Fluid (DEF) or Selective Catalytic Reduction (SCR). This was the world’s first EU Stage V soil compactor over 10-tonnes with no requirement for DEF, reducing cost and complexity for customers. The VM128D and VM138D are powered by JCB’s proven 448 diesel engine, delivering 97kW of power.

These high-torque engines drive through a 4-mode traction system, offering work, travel, gradient and PD (padfoot) shell settings. They are capable of up to a 55% gradient climbing capability. The vibratory drum offers 32Hz and 36Hz frequencies and a choice of two amplitudes: 1.8mm and 0.8mm. The two smaller models use a 256kN vibrator system and a 6,750kg front axle load, while the larger VM138D has 292kN and a 7,800kg load.

More . . . .

2/ . . .

With a 28mm drum thickness10mm stiffener rings, on the VM118D and VM128D, along with a 32mm shell thickness and 10mm rings on the VM138D, these three models deliver high vibrating masses and best-in-class durability in tough applications.

The soil compactors make use of robust, efficient mechanical hydraulic pumps, delivering fuel efficiency and durability. They are available with JCB’s INTELLICOMPACTION system and deliver Connected Compaction, through JCB LiveLink telematics. A LiveLink subscription is included with the machine for the first five years of ownership.

The 12-15 tonne machines benefit from a redesigned ROPS/FOPS cab and operator station, boasting JCB’s ergonomic design features. Flat glass all around reduces cost in the event of damage. A JCB UX 7” touchscreen display, as used on the company’s X-Series excavators provides real-time data on all machine functions and, in combination with a rotary control, provides easy access to all control menus.

The cab package includes LED front and rear working lights, four amber beacons integrated into the roof, flat glazing for easy replacement and an air conditioning and heating unit integrated into the cab roof. An optional safety pack incorporates a road lighting kit, white noise reversing alarm and an immobiliser.

ENDS 
For further information contact: Nigel Chell, JCB Press Office

Tel: 01889 593592 E-mail: nigel.chell@jcb.com  www.jcb.com